R. Konieczny, T. Walczykiewicz

Institute of Meteorology and Water Management

ul. P.Borowego 14, 30-215 Krakow

Poland

Experiences with Flood Protection Warning Systems  and Flood Risk Management Plans

## Introduction

The flood in the summer of 1997 was one of the largest in the past century in Poland. During this flood, 54 persons lost their lives; about 500 000 hectares of surface area were inundated, as well as over 12 000 businesses and institutions. About 47 500 buildings were destroyed or damaged; nearly 3 900 bridges collapsed. It is estimated that the flood of July 1997 caused losses on the order of 12 billion Polish złoty (about US$3.5 billion).

In some places, the probability of occurrence (or return period) of such a flood was about 0.1% (a so-called T1000 flood); it actually happened that culmination levels exceeded previous highest known maximums even by 2–3 meters.

In subsequent years, more floods occurred in Poland, albeit of lesser extent, but equally tragic in their effects. In 1998, a flood in the Klodzko Valley resulted in 9 fatalities. In 2001, in the Vistula basin, 18 persons died. It was necessary to evacuate about 20 000 persons, and losses reached US$600 million.

Flood disasters on the Vistula occur more frequently than on other comparable rivers. From the beginning of the 19th c. up to the 60s of the 20th c. floods have been occurring regularly on the Vistula at the rate of one in three years. The flood in August 1813 is believed to be the greatest one in the last 200 years. It caused extensive damage along the whole river.

In the 20th c. in the river-basin of the upper Vistula the most disastrous, because of their extent and intensity, were the summer floods in 1903, 1934, 1970, 1972 and 1997. Slightly less calamitous were the floods in June 1925, September 1931, May 1940, and July 1960.

Water was always the basic factor which determined the possibilities for the development and the character of  agricultural economy, livestock farming and forestry. At the same time farming and forestry bore a serious influence on the water conditions in a given area. In the early Middle Ages the whole population used to live in small settlements, grouped on the ponds, lakes, rivers or streams. With the growing number of farmsteads, the settlements were usually developing in accordance with the topography of the land and economic goals of the community, and extending up the river. This process resulted in the concentration of building development along water courses or around ponds.

## Weaknesses of the Flood Protection System

The 1997 flood was a phenomenon so enormous in magnitude that it completely overwhelmed the imagination of officials responsible for decision-making regarding the organization of flood protection activities. The existing technical infrastructure turned out to be insufficient. The capacity of retention reservoirs, relative to the catastrophic flood, was too limited, making flood wave reduction impossible. The poor condition of embankments, constructed mainly in the19th century, was the reason for many overflows. Also, the flood revealed certain equipment deficiencies of crisis intervention forces, as well as needs of units organizing flood protection, especially with regard to telecommunications, forecasting of flood damage, and warning dissemination.

Analysis of the flood process and flood damage structure leads to the conclusion that these damages were caused by:

- poor condition of technical equipment and structures, mainly embankments

- old and inefficient forecasting, precipitation and river gauge monitoring system

- faulty telecommunications system

- lack of flood warning systems

- lack of controls on flood plain development

- lack of awareness of people residing or working in flood risk areas

## Flaws

The catastrophic flood of 1997 revealed the many flaws in Poland’s flood protection system. The most significant are:

- a centralized, hierarchic structure which does not allow for dynamic local activity, especially with regard to prevention activities

- lack of warning systems for areas at risk for flash floods; the warning route within the existing structure (IMGW →WKP →PKP →GKP) makes the operation of such systems impossible by reason of their complexity

- the addressing of warnings exclusively to institutions responsible for supervising flood protection actions, without warning inhabitants and other users of the flooded areas, results in:

- lack of an efficient warning dissemination system, apart from radio and TV

- lack of an education and information system, without which people at risk for a flood do not attempt to save themselves and their possessions

- in the majority of cases, lack of prior identification of endangered areas, and thus, people and institutions in the public and private sector who should be warned.

## Actions taken - central level

After the flood in 1997, many actions were undertaken which in the future (we hope) will permit losses to be reduced in similar situations. Among the most important are changes in the law, which will introduce:

- A requirement to prepare flood risk maps for the areas most at risk

- A requirement to take flood risk zones into account in local spatial management plans and introduce limitations on development in these areas

- A requirement to secure residential structures being built in these areas (via location and building materials)

Parliament has also passed a law on natural disaster status which establishes the procedure for declaring and repealing natural disaster status, as well as operational rules for public authorities, and the scope of permissible limitations on freedom and civil rights during natural disaster status. The Law introduces a principle of one-person responsibility at the various levels of State and local government administration, as well as establishing teams to aid in the execution of their tasks.

On December 23,1997 the Republic of Poland signed a credit agreement with the International Bank for Reconstruction and Development for the implementation of the Project of the Liquidation of the Effects of Floods. The Plan for the Implementation of the Project " The Republic of Poland, Proposed Project of the Liquidation of the Effects of Floods", carried out with the assistance of the World Bank, consisting of three component parts:

- Component Part A: urban and rural infrastructure.

- Component Part B: flood protection and risk reduction

- Component Part C: project management and technical assistance

Component Part B consists of 4 subcomponents:

      - Component B.1.: Planning of flood protection in the  drainage catchment

      - Component B.2.: Monitoring, forecasting and systems of warning

      - Component B.3.: Investing in infrastructure and flood  control

      - Component  B.4:   Flood risk reduction and flood  prevention programme

Generally speaking within the framework of this project  we can discern two packages, namely:

Planning, including study works, research works, planning and design works connected with the plans of flood  control for the drainage catchments (Package I).

Upgrading monitoring networks, models, maps, equipment, including all actions designed to provide technical assistance and tools for activities  connected with the flood protection.

Within this project three parts addressed to the various levels of administration:

- Central—modernization of the nationwide flood monitoring and warning system, methodology for drawing flood maps, analysis of the possibility of introducing flood insurance

- Regional—preparation of flood damage reduction plans for the upper segments of the two main rivers in Poland (Oder and Vistula)

- Local—preparation of local plans for flood damage mitigation in municipalities, including structural and non-structural methods; preparation of a set of handbooks for local governments in the area of non-structural flood loss mitigation methods.

At the same time, actions are being undertaken whose aim is to:

- Prepare a methodology for building local flood warning systems especially for areas subject to flash flooding

- Improve collaboration with the mass media both before and during a flood (handbooks for local governments and materials for the media)

- Prepare methods and publications to facilitate conducting flood information and education campaigns at the local level.

The actions undertaken indicate a slow but important change in the way of thinking about flood damage mitigation strategy. A fundamental element of this change is the use of non-structural means of catastrophe damage mitigation, as well as propagation of participation by various entities (administration, NGO’s, the private sector and ordinary people) in flood prevention and preparation. We can hope that this is a permanent process which will enable flood losses to be reduced in the future.

General results of the World Bank Project

The basic effects of the project are as follows:

- Upgrading  of  hydro-meteorological network of  IMGW and building new system of data transfer and collection

- Improvement of modeling procedures

- Upgrading of hydro-meteorological forecasts

- The establishment and functioning of the Co-ordination and Information Centre of Flood Protection System within Regional Water Boards with the main task- cooperattion with the bodies of government administration responsible for actions connected with flood control.

- As a result of the project's implementation, particular bodies of government and self-government administration obtained reports on flood hazards in chosen valleys  coming under their spheres of activity,  which are  used in planning, consultative work and decision-making processes.

- The project provided trainings of persons employed in water boards, IMGW, as well as other persons, in the field of flood control. It also provided the publication of various informative and educational materials and guides.

## Action taken - Local Flood Warning Systems

After the flood in 1997, several flood warning systems independent of IMGW were built. Some of them were an original local initiative, some were suggested as part of the Flood Recovery Project. Some of these systems are very simple; they are more the effect of practical thinking based on experience, than of professional knowledge. Others, normally county-wide or catchment area-wide, are systems designed by consultants from universities or from IMGW. However, the aforementioned lack of experience causes flaws in all of them.

### Citizen flood warning system—Nowa Ruda

This system was prepared at the local crisis intervention center. For this reason, it is an inexpensive system, based on civic activity of inhabitants, and on modern technology. However, it also has its flaws.

Aims of system: To ‘awaken’ crisis intervention forces, to warn inhabitants.

How the system works:

- After a pre-determined water level has been exceeded, an automatic measurement station in the upper reaches of the river informs the system operator of this fact (SMS).

- The operator notifies (‘awakens’) volunteers, informing them of the necessity to begin observing water levels along the river.

- At pre-set time intervals, observers send information about water levels (SMS) to the center (database).

- The operator, based on analysis of the information sent (e.g. speed of water level growth), cloud observation, and other data from the Internet, makes the decision whether to warn inhabitants.

- Operator begins warning dissemination—a computer program distributes warnings via SMS, based on the addresses collected in the database

Advantages:

- Simplicity of solution

# Low utilization costs

- Integration of inhabitants

### Disadvantages:

- Lack of precipitation measurement

- Unreliability of SMS

### Warning system in Brzesko municipality

This warning system was prepared as part of the Flood Recovery Project. The portion concerning monitoring was prepared at IMGW in Kraków, unfortunately, further portions concerning forecasting and notification have not been implemented.

How the system works:

- Every 15 minutes, information from 3 precipitation measurement stations and 3 river gauges is sent to the database.

- If the system operator (fire brigade) notices that the water level is higher than acceptable, it makes a decision to notify inhabitants.

- Alarm sirens informing inhabitants of the hazard situation are sounded.

Advantages:

- Automatic measurement of precipitation and water levels.

Disadvantages:

- Lack of system for analysis of data facilitating the operator’s decision-making

- Lack of other warning methods aside from sirens.

### The flaws observed in local warning systems are as follows:

  - Lack of analysis concerning which areas could be at risk for inundation, and what structures are located there

  - Lack of analysis of different system variants: monitoring of precipitation and water levels (traditional, automatic), local forecasts and notification of inhabitants

  - Lack of analysis of investment and utilization costs for different system variants

  - Lack of collaboration with other entities (other municipalities, IMGW etc.)

  - Focus mainly on monitoring of water and precipitation levels, and conviction that information from this system will suffice for effective warning

  - Lack of concept how to educate inhabitants so that they will, after receiving a warning, make appropriate decisions.

Each of these flaws could cause the system not to be efficient and effective in the future.

## Summary - Activity of IMGW

In this situation, IMGW has taken several actions which should lead to an improvement in the status quo.

  - Construction of a pilot warning system jointly with the county government in Kłodzko

  - Analysis of existing local warning systems

  - Creation at IMGW of a team for collaboration with local governments, whose aim is to analyze the experiences of local governments in the area of LSOP, as well as to prepare suggestions of essential work necessary to be performed in order to help them in this

  - Preparation of educational materials to aid local activities, usable in any municipality (addressed to teachers)

  - Organization of seminars with local governments which have built or are building such systems

  - Building of a special flood portal on the Internet—www.powodz.info

  - Preparation, on the basis of previous experiences, of a handbook concerning building of local flood warning systems.